AI1276-071H SDK

(V0.0.2)

History

| Version | Date | Description | Author |
| --- | --- | --- | --- |
| V0.0.1 | 2016/06/27 | First Draft | Chia-Chang Wu |
| V0.0.2 | 2016/07/12 | Update Flash Usage | Chia-Chang Wu |

Introduction

Provide the description and programming sample of AI1276-071H SDK.

Project File

- The project file “STM32F072RBTx.uvproj” would be in the folder “STM32F072RBTx\MDK\”.

Flow Chart

- The flow chart of main() in “USER\Main.c”.

Default LoRa Settings

- The default LoRa settings are in the function “LoraPara_LoadAndConfiguration()” of “USER\Main.c”.

System Operation Mode

- Normal

  - If user doesn’t use CLI command to change System Operation Mode, Normal mode would be the default.

  - Normal Master

  - Normal Slave

  - CLI Command Flow Sample for Normal Mode:

    - COM port: 38400 baud rate, N-8-1.

    - LoraSystemMode inNormal.

    - For Master: LoraMode MASTER.

    - For Slave: LoraMode SLAVE.

    - LoraStartWork ENABLE.

    - For Master: LoraJoinNode “Slave address”.

    - If the response is not OK, please refer to “CLI_ShellCmd_LoraJoinNode()” in “USER\Cli.c” for further information.

- Product Verification

  - FSK product verification: Verify RF modulation performance.

  - LoRa product verification: Verify LoRa performance by sending and receiving specific packet.

- Ping Pong Test

  - This mode is for demonstration. The master send packet with “PING” string. The slave response packet with “PONG” string when it receives the packet from master.

CLI Command Flow Chart

- Input “LoraBW 9” to change the Bandwidth to 500kHz for example:

Flash Usage for LoRa Parameters and Data

- The functions of storing LoRa parameters and data to flash memory are in the file “USER\STM32F072RBx\Save_record.(h/c)”.

- The usage for LoRa parameters and data is defined in “USER\STM32F072RBx\Save_record.h”.

Programming Sample: LED Blink

- LED on/off by controlling specific GPIO.

- LED blinking pattern can be set by the parameters of LED blink function.

- There are three parameters of LED blink function: onTime, offTime and duration.

- Add LED Blink function to “USER\board.(h/c)” like the picture below:

  - In board.h:

  - In board.c:

- Add global variables and include “config.h” for printing debug message to console in board.c:

- Test LED Blink function by calling this function in while loop of Main.c.